# CS4023 Operating Systems

## Tutorial 10 – Memory Address Translation

Week 10 | 09–13 Nov 2026 | University of Limerick, CSIS

### Instructions

Work through these questions individually or in pairs.

### Question – Address Space Sizing

A system has: - 16-bit logical addresses - Page size = 256 bytes

- How many bits are used for the page offset?

- How many bits are used for the page number?

- How many pages are in the logical address space?

- If there are 64 frames in physical memory, what is the physical address space size (in bytes)?

- How large is each page table for a single process (assuming 2-byte page table entries)?